GOVERNMENT OF SIKKIM

Sikkim State Pollution Control Board

Sikkim State E-Waste Management Report

Reporting Period: FY 2025-26 (01 April 2025 – 31 March 2026)

Submitted under the E-Waste (Management) Rules, 2022

Submitting Authority: Sikkim State Pollution Control Board (SSPCB)

Prepared for: Central Pollution Control Board (CPCB) / Ministry of Environment, Forest and Climate Change (MoEFCC) – National E-Waste Monitoring

Date of Report Generation: 27 July 2026

This report has been compiled from the Sikkim State E-Waste Dashboard workbook maintained by SSPCB. Figures are drawn directly from the underlying data sheets; where indicated, values are illustrative/placeholder or could not be computed from the data supplied, and are flagged accordingly and should not be treated as verified departmental records pending confirmation.

# Table of Contents

1. Executive Summary	3

2. Introduction & Regulatory Context	4

3. Methodology	5

4. State E-Waste Generation Profile	7

5. Collection Performance	9

6. Formal Recycling & Treatment Infrastructure	10

7. Regulatory Compliance	11

8. Informal Sector Integration	12

9. Environmental & Health Safeguards	13

10. Awareness & Outreach	14

11. Consolidated Scorecard	15

12. Key Findings & Gap Analysis	16

13. Recommendations	17

14. Annexures – Full Raw Data Tables	18

# 1. Executive Summary

This report presents the consolidated e-waste management performance of the State of Sikkim for the reporting period FY 2025-26, compiled from the Sikkim State E-Waste Dashboard workbook maintained by the Sikkim State Pollution Control Board (SSPCB), covering generation, collection, formal recycling, regulatory compliance, informal sector integration, environmental and health safeguards, and awareness outreach, in accordance with the E-Waste (Management) Rules, 2022.

Based on the seven weighted performance categories detailed in Section 11 (Consolidated Scorecard), the State of Sikkim achieves a computed State E-Waste Management Score of 47.34 out of 100, placing it in the "Needs Improvement" performance band (Score Bands: 85-100 Excellent, 65-84 Good, 45-64 Needs Improvement, 0-44 Critical, as defined in the 0_Scorecard sheet).

Key strengths and gaps across the seven scored categories are summarised below:

- Strength – Generation & Inventory (Weight 15%): Achievement of 100%, contributing 15.00 of 15 points. 6/6 districts reporting complete (100%).

- Strength – Regulatory Compliance (Weight 15%): Achievement of 50%, contributing 7.50 of 15 points. 1 of 2 registered EPR entities compliant.

- Gap (unverified, vacuous score) – Informal Sector Integration (Weight 10%): Achievement of 100%, contributing 10.00 of 10 points, based on zero informal hotspots/workers identified in the submitted data — this reflects an absence of recorded informal-sector data rather than a confirmed field survey (see Section 8 and flags below).

- Gap (unverified, vacuous score) – Environmental & Health Safeguards (Weight 10%): Achievement of 100%, contributing 10.00 of 10 points, based on zero identified hotspots requiring monitoring — the two recorded monitoring entries are general ambient air/water stations (NAMP/NWMP), not e-waste-hotspot-specific visits, and zero e-waste health camps were conducted (see Section 9).

- Gap – Formal Recycling & Treatment (Weight 20%): Achievement of 0%, contributing 0.00 of 20 points. Sikkim has no in-state authorised recycling/dismantling facility; the sole recorded arrangement transports e-waste to West Bengal, and no material recovery rate was reported ("N/A"), so this indicator could not be computed from the data supplied (see Section 6).

- Gap – Collection Performance (Weight 20%): Achievement of only 19.18%, contributing 3.84 of 20 points. 19.18% state collection rate.

- Gap – Awareness & Outreach (Weight 10%): Achievement of only 10%, contributing 1.00 of 10 points. 2 programs conducted against a 20-programs/year benchmark.

The State Score components above are computed directly from the seven data sheets of the Sikkim State E-Waste Dashboard workbook. The workbook's own scorecard formula cells (Achieved Value / Achievement % / Score Earned) were blank in the version supplied for this report; the values shown here have been recomputed from the underlying district-, entity-, and site-level data using the category definitions and weights stated on the workbook's "Instructions" and "0_Scorecard" sheets. Two categories (Informal Sector Integration and Environmental & Health Safeguards) required a methodological judgement call because the underlying counts are zero (0/0); this is flagged explicitly in Sections 3, 8, 9, and 12. Category 3 (Formal Recycling & Treatment) could not be computed because the sole recycling arrangement reports its material recovery rate as "N/A"; it has conservatively been scored as 0% pending data, and is flagged as the single highest-priority gap in this report.

# 2. Introduction & Regulatory Context

## 2.1 Regulatory Framework

The E-Waste (Management) Rules, 2022, notified by the Ministry of Environment, Forest and Climate Change (MoEFCC), govern the environmentally sound management of electrical and electronic equipment waste in India. The Rules establish an Extended Producer Responsibility (EPR) framework under which producers of electrical and electronic equipment (EEE) are required to meet annual e-waste collection and recycling targets, either directly or through authorised Producer Responsibility Organisations (PROs), registered on the CPCB EPR Portal. The Rules also mandate registration and authorisation of recyclers, refurbishers, and dismantlers; prescribe environmentally sound management practices; and require State Pollution Control Boards (SPCBs) / Pollution Control Committees (PCCs) to monitor compliance, oversee informal sector integration, and report periodically to the CPCB and MoEFCC.

## 2.2 Scope of this Report

This report consolidates Sikkim's state-level e-waste management performance for the reporting period FY 2025-26 across seven functional areas, each corresponding to a dedicated data sheet in the Sikkim State E-Waste Dashboard workbook:

- Generation & Inventory – district-wise e-waste generation by equipment category (Sheet: 1_Generation_Inventory)

- Collection Performance – district-wise collection against generation and EPR targets (Sheet: 2_Collection_Log)

- Formal Recycling & Treatment – authorised recycler/dismantler capacity and throughput (Sheet: 3_Recycling_Infrastructure)

- Regulatory Compliance – producer/EPR entity compliance status, inspections, and penalties (Sheet: 4_Compliance_Tracker)

- Informal Sector Integration – informal cluster identification and formalisation (Sheet: 5_Informal_Sector_Hotspots)

- Environmental & Health Safeguards – site monitoring and health camps (Sheet: 6_Env_Health_Monitoring)

- Awareness & Outreach – public awareness campaigns and collection drives (Sheet: 7_Awareness_Programs)

The report is intended for submission by SSPCB to the CPCB / MoEFCC for national-level review and monitoring of state performance under the E-Waste (Management) Rules, 2022.

# 3. Methodology

## 3.1 State Score Calculation

The overall State E-Waste Management Score (out of 100) is calculated as the weighted sum of seven category-level achievement percentages, using the weights and benchmarks defined on the "0_Scorecard" sheet of the Sikkim State E-Waste Dashboard:

| # | Category | Weight | Primary Data Source Sheet |
| --- | --- | --- | --- |
| 1 | Generation & Inventory | 15% | 1_Generation_Inventory |
| 2 | Collection Performance | 20% | 2_Collection_Log |
| 3 | Formal Recycling & Treatment | 20% | 3_Recycling_Infrastructure |
| 4 | Regulatory Compliance | 15% | 4_Compliance_Tracker |
| 5 | Informal Sector Integration | 10% | 5_Informal_Sector_Hotspots |
| 6 | Environmental & Health Safeguards | 10% | 6_Env_Health_Monitoring |
| 7 | Awareness & Outreach | 10% | 7_Awareness_Programs |

Weights and category definitions reproduced exactly as entered on the "0_Scorecard" and "Instructions" sheets of the workbook.

## 3.2 Recomputation of Achieved Values

The "0_Scorecard" sheet is designed to auto-populate Achieved Value, Achievement %, and Score Earned via formulas linked to the seven data sheets. In the workbook version supplied for this report, these formula cells contained no cached values. To avoid presenting a blank or invented score, SSPCB has recomputed each category's Achieved Value directly from the underlying data, using the indicator definitions given on the "Instructions" sheet, as follows:

- Category 1 (Generation & Inventory): measured as district reporting completeness – 6 of 6 districts submitted complete category-wise generation data for the period, i.e. 100%. Note: the benchmark value entered in the workbook for this category ("5") does not share a common unit with this percentage-based indicator; SSPCB recommends this benchmark cell be corrected to a percentage or count-based target before the next reporting cycle (see Section 13).

- Category 2 (Collection Performance): measured as the state-level collection rate, i.e. total quantity collected divided by total quantity generated (Section 5).

- Category 3 (Formal Recycling & Treatment): intended to be measured as the quantity-weighted average material recovery rate achieved by authorised recyclers/dismantlers. The sole entry in the "3_Recycling_Infrastructure" sheet records its Material Recovery Rate as "N/A"; no recovery-rate figure is therefore available anywhere in the workbook for this category, and it cannot be recomputed from the data supplied. This indicator has conservatively been scored as 0% pending verified data (Section 6), rather than left blank or estimated.

- Category 4 (Regulatory Compliance): measured as the percentage of registered producers/EPR entities with a "Compliant" status (Section 7).

- Category 5 (Informal Sector Integration): intended to be measured as the simple average of (a) the percentage of identified informal hotspots that are Formalized or Partially Formalized, and (b) the percentage of estimated informal workers who are registered/trained. The "5_Informal_Sector_Hotspots" sheet records a single placeholder row ("None Identified (Sikkim Anomaly)") with an estimated-worker count of zero, i.e. zero identified hotspots and zero estimated workers (Section 8). With no hotspots or workers in the denominator, both underlying percentages are mathematically undefined (0/0) rather than zero. SSPCB has treated this vacuously as 100% achievement (nothing identified requires formalisation), but flags this explicitly as unverified: it reflects the absence of a recorded hotspot in the workbook, not a confirmed statewide field survey ruling out an informal sector (see Section 8.1 and Section 12).

- Category 6 (Environmental & Health Safeguards): intended to be measured as the percentage of identified informal hotspots that received at least one environmental monitoring visit during the period. As Section 8 records zero identified hotspots, this percentage is likewise undefined (0/0) and has been treated vacuously as 100%, flagged as unverified on the same basis (Section 9).

- Category 7 (Awareness & Outreach): measured against the benchmark of 20 programs/year entered on the "0_Scorecard" sheet; 2 programs were recorded in the "7_Awareness_Programs" sheet for the period (Section 10).

Achievement % for each category is capped at 100% and Score Earned = Weight × Achievement %. This methodology, and the resulting figures — including the two vacuous 100% scores and the single 0% data-gap score flagged above — should be verified and formally adopted by SSPCB before the score is treated as an official departmental figure.

## 3.3 Data Verification Status

Producer/EPR entity names in Section 7 (Regulatory Compliance) are recorded in the workbook using generic placeholder labels ("Entity A", "Entity B"). These are retained as-is in this report and are illustrative placeholders pending verified SSPCB/CPCB registration records; they should not be read as actual registered entity names. The recycling arrangement in Section 6 is named descriptively in the workbook ("No Local Facilities – Transported to West Bengal") rather than as a placeholder, and is retained verbatim as it appears to be a substantive statement about Sikkim's recycling infrastructure rather than a data placeholder. All numeric figures are traceable to specific rows/columns of the seven-sheet Sikkim State E-Waste Dashboard workbook, as cited in each section below.

# 4. State E-Waste Generation Profile

Source: 1_Generation_Inventory sheet. All 6 districts of Sikkim reported complete category-wise generation data for the period, updated as of 30-06-2026.

State-level total e-waste generation for the reporting period is 73,000 kg (73.00 metric tonnes), across a total reporting population of 7,49,784.

## 4.1 Category-wise Generation Breakup (State Total)

| Equipment Category | Quantity | % of State Total |
| --- | --- | --- |
| IT & Telecom | 34,500 kg (34.50 t) | 47.26% |
| Consumer Electrical & Electronics | 17,300 kg (17.30 t) | 23.70% |
| Large & Small Equipment | 8,100 kg (8.10 t) | 11.10% |
| Batteries | 6,500 kg (6.50 t) | 8.90% |
| Lighting Equipment | 3,500 kg (3.50 t) | 4.79% |
| Other | 3,100 kg (3.10 t) | 4.25% |

IT & Telecom equipment is the single largest contributing category statewide at nearly half of all generation, followed by Consumer Electrical & Electronics. Together these two categories account for 70.96% of state generation, a slightly higher concentration than seen in larger states, consistent with a small, low-industrial-base state where household IT/telecom devices and consumer electronics dominate the waste stream rather than large industrial or lighting equipment. (Described chart: a stacked/clustered bar chart of the above six categories would visually show IT & Telecom as the single dominant bar at nearly half the total, with Lighting Equipment and Other forming the smallest segments.)

## 4.2 District-wise Ranking (Highest to Lowest Generator)

| Rank | District | Population | Total Generated | Per-Capita Generation |
| --- | --- | --- | --- | --- |
| 1 | Gangtok (East Sikkim) | 2,83,583 | 42,500 kg (42.50 t) | 0.150 kg |
| 2 | Namchi (South Sikkim) | 1,46,850 | 12,100 kg (12.10 t) | 0.082 kg |
| 3 | Gyalshing (West Sikkim) | 1,36,299 | 8,500 kg (8.50 t) | 0.062 kg |
| 4 | Pakyong | 74,583 | 4,200 kg (4.20 t) | 0.056 kg |
| 5 | Soreng | 64,760 | 3,300 kg (3.30 t) | 0.051 kg |
| 6 | Mangan (North Sikkim) | 43,709 | 2,400 kg (2.40 t) | 0.055 kg |

## 4.3 Narrative Analysis

Gangtok (East Sikkim) is by a wide margin the highest-generating district in absolute terms, accounting for 58.2% of total state generation (42,500 of 73,000 kg) despite holding only 37.8% of the state's population — reflecting its status as the state capital and principal urban/commercial centre. Namchi and Gyalshing follow at considerably lower absolute volumes, while Mangan (North Sikkim), the least populous and most remote district, records the lowest absolute generation.

Unlike a per-capita-uniform generation model, per-capita generation across Sikkim's districts varies meaningfully — from 0.150 kg/person in Gangtok down to 0.051 kg/person in Soreng, roughly a 3x spread. This indicates the workbook's district-wise figures reflect genuine differentiation by urbanisation and market access rather than a flat statewide per-capita assumption. The state-average per-capita generation of 0.097 kg/person is markedly lower than typically reported for larger, more urbanised states, consistent with Sikkim's small population, low industrial base, and predominantly rural/hill-town character. This lower per-capita figure should nonetheless be corroborated against ground-level waste characterisation studies in future reporting cycles, as under-reporting in remote districts cannot be ruled out.

## 4.4 District-wise Detail (Full Category Breakup)

| District | Population | IT & Telecom (kg) | Consumer Elec. (kg) | L&S Equip. (kg) | Lighting (kg) | Batteries (kg) | Other (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gangtok (East Sikkim) | 2,83,583 | 20,000 | 10,000 | 5,000 | 2,000 | 4,000 | 1,500 |
| Namchi (South Sikkim) | 1,46,850 | 6,000 | 3,000 | 1,000 | 500 | 1,000 | 600 |
| Gyalshing (West Sikkim) | 1,36,299 | 4,000 | 2,000 | 1,000 | 500 | 600 | 400 |
| Mangan (North Sikkim) | 43,709 | 1,000 | 500 | 200 | 100 | 400 | 200 |
| Pakyong | 74,583 | 2,000 | 1,000 | 500 | 200 | 300 | 200 |
| Soreng | 64,760 | 1,500 | 800 | 400 | 200 | 200 | 200 |

# 5. Collection Performance

Source: 2_Collection_Log sheet, cross-referenced with generation figures from 1_Generation_Inventory.

State collection rate = Total Collected / Total Generated = 14,000 kg / 73,000 kg = 19.18%. State-level EPR collection achievement stands at 14,000 kg against an EPR collection target of 30,000 kg, i.e. 46.67% of target. A total of 2 registered collection centers are recorded statewide, both located in Gangtok (East Sikkim).

## 5.1 District-wise Collection Performance

| District | Generated (kg) | Collected (kg) [Rate %] | Reg. Collection Centers | EPR Target (kg) | EPR Achieved (kg) [%] |
| --- | --- | --- | --- | --- | --- |
| Gangtok (East Sikkim) | 42,500 | 8,500 (20.00%) | 2 | 20,000 | 8,500 (42.50%) |
| Namchi (South Sikkim) | 12,100 | 2,100 (17.36%) | 0 | 5,000 | 2,100 (42.00%) |
| Gyalshing (West Sikkim) | 8,500 | 1,200 (14.12%) | 0 | 2,000 | 1,200 (60.00%) |
| Mangan (North Sikkim) | 2,400 | 500 (20.83%) | 0 | 1,000 | 500 (50.00%) |
| Pakyong | 4,200 | 800 (19.05%) | 0 | 1,000 | 800 (80.00%) |
| Soreng | 3,300 | 900 (27.27%) | 0 | 1,000 | 900 (90.00%) |

## 5.2 Narrative Assessment

Against the benchmark entered on the "0_Scorecard" sheet (100% collection target), the state's achieved collection rate of 19.18% represents a shortfall of 80.82 percentage points — a substantially wider gap than typically seen in states with established decentralised collection networks.

Five of Sikkim's six districts (Namchi, Gyalshing, Mangan, Pakyong, Soreng) record zero registered collection centers. Only Gangtok reports any dedicated collection infrastructure (2 centers). The Collection Log's Remarks field, populated for only Gangtok ("Mostly institutional") and Namchi ("Limited household collection"), contains no equivalent of a statewide coordinating body or door-to-door collection scheme (comparable to Kerala's Haritha Karma Sena / Clean Kerala Company Limited model); no such mechanism is recorded anywhere in this workbook.

Notwithstanding the low absolute collection rate, EPR achievement-against-target varies widely by district: Soreng (90.00%) and Pakyong (80.00%) both substantially exceed Gangtok's EPR achievement (42.50%) and Namchi's (42.00%), despite having no registered collection centers of their own — suggesting their EPR targets have been set at a level low enough to be met through channels outside formal local collection infrastructure, rather than reflecting stronger underlying collection performance.

# 6. Formal Recycling & Treatment Infrastructure

Source: 3_Recycling_Infrastructure sheet.

| Recycler / Dismantler | Authorisation No. | District | Installed Capacity (t/yr) | Processed This Year (t) | Material Recovery |
| --- | --- | --- | --- | --- | --- |
| No Local Facilities – Transported to West Bengal | SK/WB-Auth/2026 | All Districts | 100 | 24.1 | N/A |

Sikkim has no in-state CPCB/SPCB-authorised e-waste recycling or dismantling facility. The single entry in this sheet is an out-of-state transport arrangement (to West Bengal); its "Installed Capacity" figure (100 t/yr) most likely reflects an allocated transport/processing quota under that inter-state arrangement rather than dedicated in-state infrastructure, and its Material Recovery Rate is recorded as "N/A" — not measured or not disclosed. This makes Category 3 (Formal Recycling & Treatment) the single least-verifiable and lowest-scoring category in this report (see Sections 3.2 and 11); SSPCB should obtain a verified recovery-rate figure from the West Bengal facility handling Sikkim's e-waste before the next reporting cycle.

## 6.1 Capacity vs. Generation Adequacy

Total nominal recycling/processing capacity associated with Sikkim's e-waste (via the West Bengal arrangement) is 100 tonnes/year, against total state e-waste generation of 73.00 tonnes/year for the period – a nominal capacity-to-generation ratio of 136.99%. Total quantity actually processed under this arrangement this year is 24.1 tonnes, or 33.01% of total state generation. Unlike the capacity figure, which nominally exceeds generation, actual throughput remains well below the volume of e-waste generated statewide — indicating that, notwithstanding the capacity on paper, the majority of Sikkim's generated e-waste (66.99%) is not confirmed to have reached a formal, authorised recycling channel during the period.

## 6.2 Material Recovery Performance

No material recovery rate figure is available in the workbook for Sikkim's sole recycling arrangement (recorded as "N/A"). Consequently, neither a simple-average nor a quantity-weighted-average material recovery rate can be computed for the state — in contrast to states with multiple in-state authorised facilities where such an average can be derived directly from reported recovery rates. This is recorded as a critical data gap rather than a zero-recovery finding, and is the primary driver of Category 3's 0% score in Section 11.

# 7. Regulatory Compliance

Source: 4_Compliance_Tracker sheet. 2 registered producers/EPR entities are recorded for the state.

| Metric | Value |
| --- | --- |
| Total Registered Producers / EPR Entities | 2 |
| Compliant | 1 (50%) |
| Partially Compliant | 0 (0%) |
| Non-Compliant | 1 (50%) |
| Inspections Planned | 2 |
| Inspections Conducted | 1 (50.0% of planned) |
| Penalties / Show-Cause Issued | 1 |
| Penalties Resolved (against issued penalties) | 0 (0.0% of issued) |

Entity names "Entity A" and "Entity B" are placeholder identifiers pending verified SSPCB/CPCB EPR registration records and should not be presented as final registered entity names in the national submission.

## 7.1 Compliance Status by Entity

| Entity | EPR Reg. No. | Status | Inspection / Penalty Status | Remarks |
| --- | --- | --- | --- | --- |
| Entity A | EPR/SK/001 | Compliant | Planned: Yes / Conducted: Yes / Penalty Issued: No / Penalty Resolved: Yes* | — |
| Entity B | EPR/SK/002 | Non-Compliant | Planned: Yes / Conducted: No / Penalty Issued: Yes / Penalty Resolved: No | Notice served |

Entity A's record shows "Penalty Resolved = Yes" while its "Penalty / Show-Cause Issued" field is recorded as "No" — i.e. a penalty is marked resolved despite no penalty having been issued to this entity. This internal inconsistency should be corrected at source before the next submission. The single genuine issued-and-unresolved penalty in the state is against Entity B (Non-Compliant, notice served, inspection not yet conducted), which is the basis for the 0.0%-of-issued penalties-resolved figure above.

# 8. Informal Sector Integration

Source: 5_Informal_Sector_Hotspots sheet.

| Hotspot / Cluster | District | Status | Est. Workers | Workers Registered/Trained | Formalization Action | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| None Identified (Sikkim Anomaly) | All | N/A | 0 | 0 | N/A | Zero informal workers observed |

## 8.1 Analysis

The workbook records zero identified informal e-waste clusters and zero estimated informal workers statewide, in a single placeholder row explicitly labelled "Sikkim Anomaly". No formalisation actions, remediation plans, or worker-training records are therefore applicable.

A statewide reading of zero informal e-waste hotspots is plausible given Sikkim's small population, low industrial base, and predominantly rural/hill-town geography, and may genuinely differ from larger, more urbanised states. However, the single-row placeholder entry (rather than a documented set of surveyed sites, each explicitly ruled out, as seen in other states' hotspot sheets) does not by itself demonstrate that a systematic field survey or informal-sector mapping exercise was carried out. SSPCB should confirm whether this reflects (a) a completed statewide informal-sector survey that found no clusters, or (b) the absence of such a survey to date. Category 5's 100% achievement score in Section 11 is scored vacuously (0 identified / 0 required to formalise) and should be treated as unverified until this is confirmed.

# 9. Environmental & Health Safeguards

Source: 6_Env_Health_Monitoring sheet.

| Site / Station | District | Monitoring Type | Date | Result | Health Camp | Beneficiaries |
| --- | --- | --- | --- | --- | --- | --- |
| NAMP-SK-01 Deorali | Gangtok | Air Quality | 10-06-2026 | Normal | No | 0 |
| NWMP-SK-01 Teesta | Namchi | Water Quality | 15-06-2026 | Normal | No | 0 |

## 9.1 Analysis

2 distinct environmental monitoring visits are recorded for the period, both returning "Normal" results. Neither visit was accompanied by a health camp; total health camps conducted statewide is 0, and total beneficiaries screened is 0.

The two recorded sites (NAMP-SK-01 Deorali and NWMP-SK-01 Teesta) are general National Air Monitoring Programme (NAMP) / National Water Monitoring Programme (NWMP) ambient-quality stations, not monitoring visits tied to a specific identified e-waste informal hotspot — consistent with Section 8 recording zero such hotspots. Zero e-waste-specific health camps and zero beneficiaries were screened statewide during the period.

# 10. Awareness & Outreach

Source: 7_Awareness_Programs sheet.

| Program Name | District | Date | Type | Target Audience | Participants |
| --- | --- | --- | --- | --- | --- |
| Clean Sikkim E-Waste Free | Gangtok | 12-05-2026 | Workshop | General Public | 5,000 |
| School E-Waste Drive | Namchi | 20-05-2026 | Collection Drive | Students | 2,400 |

A total of 2 awareness/outreach programs were conducted statewide during the period, against the benchmark target of 20 programs/year set on the "0_Scorecard" sheet – an achievement of 10.0% of target. Of these, 1 was a school-based collection drive. A cumulative 7,400 participants were reached statewide across both programs, driven predominantly by the single large-format "Clean Sikkim E-Waste Free" workshop in Gangtok (5,000 participants).

# 11. Consolidated Scorecard

Reproduced/recomputed from the 0_Scorecard sheet per the methodology in Section 3.

| # | Category | Weight | Benchmark / Target | Achieved Value | Achievement % | Score Earned |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Generation & Inventory | 15% | 5 (unit not specified in workbook) | 6/6 districts reporting complete (100%) | 100% | 15.00 |
| 2 | Collection Performance | 20% | 1 (i.e., 100% collection target) | 19.18% state collection rate | 19.18% | 3.84 |
| 3 | Formal Recycling & Treatment | 20% | 1 (i.e., 100% material recovery target) | Recovery rate "N/A" – not computable from data supplied | 0%* | 0.00* |
| 4 | Regulatory Compliance | 15% | 1 (i.e., 100% compliance target) | 50.0% entities compliant | 50% | 7.50 |
| 5 | Informal Sector Integration | 10% | 1 (i.e., 100% formalization target) | 0 hotspots / 0 workers identified (vacuous) | 100%* | 10.00* |
| 6 | Environmental & Health Safeguards | 10% | 1 (i.e., 100% monitoring coverage target) | 0 hotspots identified to monitor (vacuous) | 100%* | 10.00* |
| 7 | Awareness & Outreach | 10% | 20 programs/year | 2 programs conducted | 10% | 1.00 |

TOTAL STATE SCORE: 47.34 / 100 – Performance Band: Needs Improvement

Score Bands (per 0_Scorecard sheet): 85-100 Excellent | 65-84 Good | 45-64 Needs Improvement | 0-44 Critical.

Rows marked "*" (Categories 3, 5, and 6) rest on judgement calls explained in Sections 3, 6, 8, and 9 rather than directly reported percentages: Category 3 is scored 0% because no recovery rate could be computed (a conservative, data-gap-driven score, not a measured zero); Categories 5 and 6 are scored 100% vacuously because their denominators (identified hotspots/workers) are zero in the data supplied. If Category 3 were instead excluded pending data and the remaining six categories re-weighted proportionally, or if Categories 5–6 were instead treated as not-yet-verified/excluded, the total score would differ materially from 47.34.

# 12. Key Findings & Gap Analysis

## 12.1 Strongest-Scoring Categories

- Generation & Inventory (Section 4): 100% of districts (6/6) reported complete data, reflecting strong data-collection discipline across a small state, though the benchmark unit inconsistency noted in Section 3.2 should be corrected.

- Regulatory Compliance (Section 7): 1 of 2 registered producers/EPR entities is Compliant (50%), with the single non-compliant entity already under active penalty proceedings.

Two further categories (Informal Sector Integration and Environmental & Health Safeguards) show 100% achievement in Section 11, but as set out above and repeated here for emphasis, these are vacuous scores arising from zero recorded hotspots/workers rather than demonstrated strong performance, and should not be read as strengths without further verification.

## 12.2 Weakest-Scoring Categories

- Formal Recycling & Treatment (Section 6): the single largest and most unambiguous gap in this report. Sikkim has no in-state authorised recycling/dismantling facility; e-waste is transported to West Bengal under an arrangement for which no material recovery rate is reported, and actual processed volume (24.1 t) covers only 33.01% of state generation (73.00 t).

- Collection Performance (Section 5): the state collection rate of 19.18% is well below the 100% benchmark and well below EPR-target achievement of 46.67%, with 5 of 6 districts recording zero registered collection centers.

- Awareness & Outreach (Section 10): only 2 of a 20-program annual benchmark were conducted (10.0%), the lowest achievement percentage among the categories with directly measurable (non-vacuous) data.

- Regulatory Compliance data quality (Section 7): only 1 of 2 planned inspections was conducted (50%), and the single entity marked "Penalty Resolved = Yes" without a corresponding issued penalty is an internal data inconsistency requiring correction.

## 12.3 Data Quality & Verification Gaps

- The absence of any recorded informal-sector hotspots or workers (Section 8) and the general, non-hotspot-specific nature of the two environmental monitoring visits (Section 9) mean two of the seven scorecard categories cannot be verified as genuine strong performance versus an unsurveyed data gap.

- The sole recycling/dismantling entry's material recovery rate is recorded as "N/A", leaving Category 3 without a computable Achievement % and driving nearly half of the state's total score shortfall on its own (20% weight, scored 0%).

# 13. Recommendations

- 1. Establish or formally contract in-state (or dedicated cross-border) authorised recycling/dismantling capacity, and obtain a verified material recovery rate from the West Bengal facility currently processing Sikkim's e-waste, so that Category 3 can be scored from actual data rather than treated as a 0% data gap.

- 2. Expand registered collection centers beyond Gangtok into the five districts (Namchi, Gyalshing, Mangan, Pakyong, Soreng) currently recording zero centers, and consider a decentralised, community-based collection mechanism to raise the state collection rate from 19.18% toward the 100% benchmark.

- 3. Scale up awareness and outreach programming substantially – current performance (2 of a 20-program annual target) is the weakest directly-measured category; SSPCB should set a district-wise annual awareness-campaign calendar, particularly extending outreach to the four districts (Gyalshing, Mangan, Pakyong, Soreng) with no recorded programs in the period.

- 4. Commission and document a statewide informal e-waste sector survey to confirm whether the zero-hotspot finding in Section 8 reflects a genuine absence or an unsurveyed gap, and update the 5_Informal_Sector_Hotspots sheet with dated, site-level survey records rather than a single placeholder row.

- 5. Conduct at least one e-waste-hotspot-specific environmental monitoring visit and health camp, distinct from the general NAMP/NWMP ambient stations currently recorded, once/if informal or formal e-waste handling sites are identified under Recommendation 4.

- 6. Complete the 1 pending inspection (of 2 planned) in the Compliance Tracker, and pursue time-bound resolution of the 1 outstanding penalty/show-cause proceeding against the non-compliant EPR entity (Section 7).

- 7. Correct the internally inconsistent compliance record for Entity A ("Penalty Resolved = Yes" against "Penalty Issued = No") ahead of the next submission.

- 8. Correct the benchmark/target value entered for the Generation & Inventory category on the "0_Scorecard" sheet, which is not currently expressed in a unit consistent with its associated indicator, so that future scorecards compute this category's Achievement % without manual reinterpretation.

- 9. Adopt and document an explicit scorecard convention for zero-denominator categories (Informal Sector Integration, Environmental & Health Safeguards) and for missing-indicator categories (Formal Recycling & Treatment), so that future scores are computed consistently rather than requiring case-by-case judgement calls as in this report.

- 10. Replace placeholder producer/EPR entity names in the Compliance Tracker with verified SSPCB/CPCB registration records ahead of the next formal submission to CPCB/MoEFCC.

# 14. Annexures – Full Raw Data Tables

The following tables reproduce the complete district-level and entity-level data from all seven sheets of the Sikkim State E-Waste Dashboard workbook, for verification and audit purposes.

## Annexure A – Sheet 1: Generation Inventory (Full)

| District | Population | IT & Telecom (kg) | Consumer Elec. (kg) | L&S Equip. (kg) | Lighting (kg) | Batteries (kg) | Other (kg) | Total (kg) | Date Updated |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gangtok (East Sikkim) | 2,83,583 | 20,000 | 10,000 | 5,000 | 2,000 | 4,000 | 1,500 | 42,500 | 30-06-2026 |
| Namchi (South Sikkim) | 1,46,850 | 6,000 | 3,000 | 1,000 | 500 | 1,000 | 600 | 12,100 | 30-06-2026 |
| Gyalshing (West Sikkim) | 1,36,299 | 4,000 | 2,000 | 1,000 | 500 | 600 | 400 | 8,500 | 30-06-2026 |
| Mangan (North Sikkim) | 43,709 | 1,000 | 500 | 200 | 100 | 400 | 200 | 2,400 | 30-06-2026 |
| Pakyong | 74,583 | 2,000 | 1,000 | 500 | 200 | 300 | 200 | 4,200 | 30-06-2026 |
| Soreng | 64,760 | 1,500 | 800 | 400 | 200 | 200 | 200 | 3,300 | 30-06-2026 |
| STATE TOTAL | 7,49,784 | 34,500 | 17,300 | 8,100 | 3,500 | 6,500 | 3,100 | 73,000 | — |

## Annexure B – Sheet 2: Collection Log (Full)

| District | Generated (kg) | Collected (kg) | Reg. Centers | EPR Target (kg) | EPR Achieved (kg) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Gangtok (East Sikkim) | 42,500 | 8,500 | 2 | 20,000 | 8,500 | Mostly institutional |
| Namchi (South Sikkim) | 12,100 | 2,100 | 0 | 5,000 | 2,100 | Limited household collection |
| Gyalshing (West Sikkim) | 8,500 | 1,200 | 0 | 2,000 | 1,200 | — |
| Mangan (North Sikkim) | 2,400 | 500 | 0 | 1,000 | 500 | — |
| Pakyong | 4,200 | 800 | 0 | 1,000 | 800 | — |
| Soreng | 3,300 | 900 | 0 | 1,000 | 900 | — |
| STATE TOTAL | 73,000 | 14,000 | 2 | 30,000 | 14,000 | — |

## Annexure C – Sheet 3: Recycling Infrastructure (Full)

| Recycler/Dismantler | Authorisation No. | District | Installed Cap. (t/yr) | Processed (t) | Recovery Rate | Valid Till |
| --- | --- | --- | --- | --- | --- | --- |
| No Local Facilities – Transported to West Bengal | SK/WB-Auth/2026 | All Districts | 100 | 24.1 | N/A | 31-12-2027 |
| STATE TOTAL | — | — | 100 | 24.1 | N/A | — |

## Annexure D – Sheet 4: Compliance Tracker (Full)

| Entity | Reg. No. | Status | Insp. Planned | Insp. Conducted | Penalty Issued | Penalty Resolved | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Entity A | EPR/SK/001 | Compliant | Y | Y | N | Y | — |
| Entity B | EPR/SK/002 | Non-Compliant | Y | N | Y | N | Notice served |

## Annexure E – Sheet 5: Informal Sector Hotspots (Full)

| Cluster | District | Status | Est. Workers | Workers Reg. | Formalization Action | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| None Identified (Sikkim Anomaly) | All | N/A | 0 | 0 | N/A | Zero informal workers observed |

## Annexure F – Sheet 6: Environmental & Health Monitoring (Full)

| Site | District | Type | Date | Result | Health Camp | Beneficiaries | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NAMP-SK-01 Deorali | Gangtok | Air Quality | 10-06-2026 | Normal | N | 0 | General Station |
| NWMP-SK-01 Teesta | Namchi | Water Quality | 15-06-2026 | Normal | N | 0 | General Station |

## Annexure G – Sheet 7: Awareness Programs (Full)

| Program | District | Date | Type | Audience | Participants |
| --- | --- | --- | --- | --- | --- |
| Clean Sikkim E-Waste Free | Gangtok | 12-05-2026 | Workshop | General Public | 5,000 |
| School E-Waste Drive | Namchi | 20-05-2026 | Collection Drive | Students | 2,400 |
| TOTAL | — | — | — | — | 7,400 |

## Annexure H – Sheet 0: Scorecard Weights & Benchmarks (as entered)

| # | Category | Weight | Benchmark/Target (as entered) |
| --- | --- | --- | --- |
| 1 | Generation & Inventory | 15% | 5 |
| 2 | Collection Performance | 20% | 1 |
| 3 | Formal Recycling & Treatment | 20% | 1 |
| 4 | Regulatory Compliance | 15% | 1 |
| 5 | Informal Sector Integration | 10% | 1 |
| 6 | Environmental & Health Safeguards | 10% | 1 |
| 7 | Awareness & Outreach | 10% | 20 |